Witt3D_GIS - User Manual (English)

1. Introduction & Application Overview

Witt3D_GIS is a mobile Geographic Information System (GIS) and High-Precision Surveying application designed for field engineers, surveyors, and GIS specialists.

Key Capabilities

High-Precision GNSS Positioning: Connects to external GNSS receivers (e.g., u-blox F9P) via USB OTG with native support for NMEA sentences and binary UBX NAV-PVT protocols.

NTRIP RTK Differential Correction Client: Built-in NTRIP client streams RTCM3 corrections from reference networks to achieve centimeter- or millimeter-level RTK positioning (Fix/Float status).

Interactive Map & Layer Management: Renders vector tiles, Google Earth Hybrid satellite imagery, high-contrast debug basemaps, and custom external Web Map Service (WMS) layers up to extreme zoom scales (down to 1:5 scale / zoom level 24.0+).

Feature Digitizing & Editing: Create and edit Points, LineStrings, and Polygons with snapping capabilities, metadata notes, interactive map relocation, and precise relocation by exact distance and bearing angle.

Measurement & Target Navigation: Distance measurement tool with a dynamic off-screen target arrow pointing towards active survey points with real-time distance updates.

Multi-Format GIS Import & Export: Export collected field data directly to ESRI Shapefile ZIP bundles (.shp, .shx, .dbf), GeoJSON, KML, QGZ, and CSV formats.

Bilingual UI: Native support for English and German UI localizations.

2. Screen Layout & Interface Overview

The interface is organized into four main functional regions:

2.1 Top Status & Control Bar

Baud Rate Spinner: Dropdown list to select GNSS serial baud rates (4800 to 921600 bps; default: 115200).

Connect / Disconnect Button: Initiates or terminates the serial connection to the USB GNSS hardware.

RTK Fix Status Indicator: Displays real-time GNSS fix quality (NO FIX, SINGLE, FLOAT, or FIX).

Sats Count (`Sats: X`): Shows the current number of tracked positioning satellites.

Age of Differential (`Age: X`): Shows the age of RTCM differential corrections in seconds.

Altitude (`Alt: X`): Displays real-time orthometric/ellipsoidal height in meters.

Follow Button (`Follow`): Centers and locks the map view onto the live GNSS crosshair position.

Measure Button (`Meas`): Toggles interactive distance measurement mode.

Set Point Button (`Set`): Sets the current map center or GNSS coordinate as the active measurement target.

2.2 Secondary Drawing Toolbar

Located immediately below the top status bar:

`Draw Pt` (Draw Point): Activates manual point creation by tapping on the map.

`Add Pt @ GNSS`: Instantly creates a point feature at the exact current GNSS location.

`Draw Line`: Begins or completes drawing a multi-segment LineString feature.

`Draw Poly`: Begins or completes drawing a closed Polygon feature.

`Edit`: Toggles feature editing mode (tap any existing object to edit or delete).

`Clear`: Clears active drawing points or distance measurement vectors.

`Snap` (Magnet Icon): Toggles snapping mode to automatically snap new vertices to existing geometry points within a 10m radius.

Active Mode Styling: When a tool is active, the corresponding button highlights in RAL 5005 Signal Blue (#00538E) with white text.

2.3 Map Canvas & Overlay HUD

Center Coordinates HUD: Real-time latitude and longitude of the map center displayed in the bottom-right corner (updated continuously during map panning/zooming).

Scale Bar & Relative Scale: Displays an accurate MapLibre scale bar in the top-left corner alongside the dynamic relative scale ratio (e.g., 1:250).

Off-Screen Distance Arrow: When a measurement target point is navigated off-screen, an edge arrow appears on the screen boundary pointing toward the target with live distance updates in meters.

2.4 Left Navigation Drawer (GIS Menu)

Accessed via the hamburger menu icon in the top-left corner:

GIS Menu / Layers: Toggle visibility of GNSS Crosshair and Manual Geometries.

WMS Layers: Add, edit, or toggle external raster WMS layers.

Import Data: Import CSV or GIS files (.shp, .json, .qgz, .kml).

Export Data: Export field data in Shapefile ZIP, GeoJSON, KML, QGZ, or CSV format.

Display Settings: Toggle Relative Scale indicator, Diagnostic Isolation Mode, or switch app language between English and German.

Manual Geometries List: An expandable tree list of all digitized features with names and notes. Tapping any feature zooms the map directly to its location (zoomToFeature).

3. GNSS Receiver Connection & Hardware Setup

3.1 USB OTG Hardware Connection

Connect your external GNSS receiver (e.g., u-blox ZED-F9P via USB OTG adapter) to your device.

Grant the USB device permission prompt when requested.

Select the receiver's serial baud rate from the top dropdown list (Default: 115200).

Tap Connect. Upon successful connection, the status banner updates to Serial Connected.

3.2 Dual Protocol Support (NMEA & UBX NAV-PVT)

Witt3D_GIS automatically parses standard NMEA GGA/RMC sentences as well as high-frequency binary u-blox UBX NAV-PVT packets.

Binary UBX data provides millisecond-accurate timestamping, precise RTK fix determination, carrier phase solution tracking, and raw satellite count.

3.3 Skyplot & Satellite Signal Status

Open the Navigation Drawer and tap Settings / NTRIP Settings.

Tap the Skyplot button.

The Skyplot window renders a polar chart showing satellite positions (Azimuth and Elevation) along with PRN identifiers and Signal-to-Noise Ratio (SNR) signal strength bars.

4. NTRIP RTK Differential Correction Setup

To achieve centimeter-level positioning accuracy, configure the built-in NTRIP client:

Open the left menu and select Settings -> NTRIP Settings.

Enter your RTK correction provider details:

NTRIP Host: IP address or hostname of the caster (e.g., rtk.provider.com).

NTRIP Port: Caster port (typically 2101 or 8080).

NTRIP Mountpoint: Mountpoint identifier (e.g., VRS_3_2_DE).

NTRIP User: Your account username.

NTRIP Pass: Your account password.

Manual Position Option: If live position is unavailable, check Manual Position to send static GGA coordinates to the caster.

Tap Save & Start NTRIP.

Once connected, RTCM3 correction data streams directly over the socket to your receiver. Watch the top status bar transition from SINGLE -> FLOAT -> `FIX`.

5. Map Styles & Raster WMS Layers

5.1 Basemap Selection

The application includes multiple pre-configured map styles:

Google Earth Hybrid (Default): High-resolution satellite imagery overlaid with road network and place labels.

High Contrast (Yellow): Bright yellow background (#FFFF00) optimized for extreme sunlight visibility and geometry isolation debugging.

Standard vector street map styles.

5.2 WMS (Web Map Service) Configuration

Open the left menu and tap WMS Layers.

Tap Add WMS Layer.

Enter the WMS endpoint URL (e.g., https://geodaten.land.de/wms) and layer names (comma-separated).

Save the configuration. External WMS layers are rendered beneath manual vector geometries at max zoom levels up to 35.0f, ensuring raster imagery never disappears when zooming in close.

6. Digitizing & Editing Geometries

6.1 Creating Points

Manual Map Tap (`Draw Pt`): Tap Draw Pt, then tap anywhere on the map. Enter a feature Name and Notes, then tap Save.

Instant Point at Receiver (`Add Pt @ GNSS`): Tap Add Pt @ GNSS. A point feature is immediately created at the current GNSS coordinate.

6.2 Drawing Lines & Polygons

Tap Draw Line or Draw Poly.

Tap sequential locations on the map to place vertices. A temporary red line or polygon is rendered in real time.

Tap Draw Line or Draw Poly again to finish digitizing.

Enter the Name and Notes in the prompt dialog and tap Save. Saved features render in Magenta.

6.3 Vertex Snapping Mode (Snap)

Tap the magnet icon (Snap) on the drawing bar to enable snapping mode.

When placing a new point or moving a vertex, the cursor automatically snaps to the exact coordinates of any existing feature vertex within a 10-meter radius.

6.4 Editing Features

Tap the Edit button on the drawing toolbar.

Tap any feature on the map to open the Feature Properties Dialog.

Available Actions:

Save: Update Name and Notes.

Delete: Permanently remove the feature.

Move: Tap Move, then tap a new position on the map to relocate the object.

Move By...: Enter an exact distance (meters) and bearing angle (0–360°) to shift the feature with millimeter precision.

7. Measurement Tool & Target Navigation

Tap Meas on the top bar to enter Measurement Mode.

Tap points on the map to measure linear distances between vertices.

Tap Set to establish a survey target point.

Off-Screen Target Arrow: If you navigate or walk away from the target point such that it leaves the screen, a dynamic navigation arrow (ic_nav_arrow) appears on the screen boundary pointing toward the target and displaying real-time distance in meters.

8. Data Import & Export

8.1 Importing GIS Data

Open the left menu and tap Import Data.

Select Import CSV (Name, Lat, Lon) or Import GIS File (.shp, .json, .qgz, .kml).

Browse and select the file using the system file picker. Features are imported and displayed directly on the map.

8.2 Exporting GIS Data

Open the left menu and tap Export Data.

Choose your preferred export format:

Shapefile (.zip): Generates a complete ZIP archive containing .shp, .shx, and .dbf files formatted to ESRI standards.

GeoJSON (.json)

KML (.kml)

QGIS Project (.qgz)

Use the system file dialog to save the exported file to your device or external SD card.

9. Language & Display Settings

Language Switcher: Open the left menu and tap Language (DE / EN) to toggle the interface between German and English. Setting is saved in app preferences.

Relative Scale Indicator: Displays dynamic real-time map scale (e.g., 1:500) recalculated based on screen pixel density and zoom level.